Harold’s Circuits: Flip-Flops

Cheat Sheet

4 March 2025

| S-R Flip-Flop (Edge-Triggered) |  |  |
| --- | --- | --- |
| Style | NAND-NAND | AND-NOR |
| Circuit |  |  |
| Symbol |  |  |
| Truth Table | S; R; Qnext; Action. 0; 0; Q; No change, Hold. 0; 1; 0; Reset (Q 0). 1; 0; 1; Set (Q 1). 1; 1; X; Invalid, Not allowed. |
| Boolean Equation | Qnext=RQprev+RS=R(Qprev+S) |  |
| Name Origin | SR for Set-Reset |  |
| Observations | - A Flip-Flop is a Latch with 2 AND/NAND gates added for clock input to trigger data flow from left to right |  |
| Applications | - Storing a single bit of data, 1 or 0 |  |
| TTL Chips | 74x71, 74Lx74 |  |
| D Flip-Flop (Edge-Triggered) |  |  |
| Style | NAND-NAND | AND-NOR |
| Circuit |  |  |
| Symbol |  |  |
| Truth Table | Inputs; Outputs; Action. D; CLK; Qnext; Q’next. 0; ↑; 0; 1; Reset (Q 0). 1; ↑; 1; 0; Set (Q 1). |
| Boolean Equation | Qnext=D |  |
| Name Origin | D for Delays, since it delays the signal until the next active clock transition |  |
| Observations | - Made with S-R flip-flop with input S inverted for input R - Stores a single bit after the edge-triggered clock pulse |  |
| Applications | - Storing Bits (memory) in a pipeline - Event Detection |  |
| TTL Chips | 74x74, 74x79, 74x171, 74x173 |  |

| J-K Flip-Flop (Edge-Triggered) |  |  |
| --- | --- | --- |
| Style | NAND-NAND | AND-NOR |
| Circuit |  |  |
| Symbol |  |  |
| Truth Table | Inputs; Outputs. J; K; CLK; Qnext; Q’next; Action. 0; 0; ↓; Q; Q’; Hold, No change. 0; 1; ↓; 0; 1; Reset (Q 0). 1; 0; ↓; 1; 0; Set (Q 1). 1; 1; ↓; Q’; Q; Toggle, Change (1 0). |
| Boolean Equation | Qnext=JQ prev+KQprev |  |
| Name Origin | None, other than J and K are adjacent letters in the alphabet |  |
| Observations | - Same as S-R flip-flop except 2 feedback lines added - Fixes the invalid 1-1 state |  |
| Applications | - Frequency Division: If J=K=HIGH, then clock frequency divider f2 - Counting: If cascaded with QA wired to JKB CLK, then QA = LSB and QB=MSB - Sequence Detection: If cascaded with QAJB and Q’AKB, then tap Q/Q’s for 1/0 pattern, then AND for output |  |
| TTL Chips | 74x68, 74x69, 74x70, 74x73, 74x76, 74x101, 74x102, 74X103, 74x107 |  |
| T Flip-Flop (Edge-Triggered) |  |  |
| Style | NAND-NAND | AND-NOR |
| Circuit |  |  |
| Symbol |  |  |
| Truth Table | Inputs; Outputs; Action. T; CLK; Qnext; Q’next. 0; ↑; Q; Q’; No change, Hold. 1; ↑; Q’; Q; Change, Toggle. |
| Boolean Equation | Qnext=Qprev |  |
| Name Origin | T for Toggle, since it changes state on the triggering edge of the clock pulse |  |
| Observations | - Made with J-K flip-flop with input T connected to both J and K - Implements the two middle rows of the J-K flip-flop truth table |  |
| Applications | - Frequency Division: If J=K=HIGH, then clock frequency divider f2 |  |
| TTL Chips | 74x374 or use J-K flip-flop chips |  |

| J-K Flip-Flop Applications |  |
| --- | --- |
| Frequency Division | If J=K=HIGH, then clock frequency divider f2 |
| Counting | If cascaded with QA wired to JKB CLK, then QA = LSB and QB=MSB |

| Sequence Detection | If cascaded with QAJB and Q’AKB, then tap Q/Q’s for 1/0 pattern, then AND for output |
| --- | --- |

Source:

Diagrams came from “ECPI University EET 230 – Digital Systems II”, Wikipedia, and Google images.